HMI-Based Automated Warehouse Control System

Table of Contents

Inroduction	3

HMI Screen Design	3

Root Screen (Mode Selection)	3

Automatic Mode Screen	4

HMI Project Setup in TIA Portal	8

Adding a New HMI Screen	8

Add a New Screen	10

Create Navigation Buttons	11

Editing "Automatic" Screen (Screen_1)	13

Add a Start Button	13

Add a Running Indicator (Lamp)	15

Display Target Position (I/O Field)	16

Add a Navigation Button to Return to the Root Screen	17

# Inroduction

This document provides a step-by-step guide to designing an HMI (Human-Machine Interface) system for an automated warehouse using Siemens TIA Portal. It covers setting up an HMI screen, configuring buttons, lamps, and numeric displays to control and monitor the system.

## HMI Screen Design

### Root Screen (Mode Selection)

- Two selection buttons:

  - Automatic Mode

  - Manual Mode

### Automatic Mode Screen

- Start Button: Activates warehouse automation.

- Stop Button: Stops the process.

- Status Lamp:

  - Green when the system is running.

  - Red when stopped.

- Display of the target position of the system.

2.System Operation in TIA Portal

1. Startup and Mode Selection

- When the system starts, the Root Screen appears on the HMI.

- The user selects Automatic Mode, which navigates to a new screen containing:

  - Start Button

  - Stop Button

  - Lamp (Status Indicator)

  - Target Position Display

2. Start and Stop Button Logic

2.1. Start Button Behavior

When the Start button is pressed:

- The Start Lamp (HMI Lamp) turns Green.

- A Start Light is set in the PLC.

- The Warehouse Automation Process begins.

2.2. Stop Button Behavior

When the Stop button is pressed:

- The Start Lamp (HMI Lamp) turns Red.

- The Start Light is reset in the PLC and the Stop Light is set.

- The Warehouse Automation Process stops.

The Target Position Screen on the HMI is used to display the current target position of the automated warehouse.

# HMI Project Setup in TIA Portal

## Adding a New HMI Screen

1.Add a New Device

- In the Project Tree window, click Add New Device.

- Select the appropriate HMI model, ensuring compatibility with your PLC.

- Verify that the HMI supports the required communication protocol (e.g., Profinet, Profibus, Modbus).

2.Connect HMI to the PLC

- Establish a connection between the HMI and PLC within TIA Portal.

This is the Screen you will get after clicking finish.

3.Adding and Navigating Between Screens

### Add a New Screen

- From the HMI dropdown menu, select Add New Screen.

- Name the screen appropriately based on its function (e.g., "Automatic").

## Create Navigation Buttons

Method 1:

  - Select a button from the Elements menu.

  - Right-click on the button and choose Properties.

  - Go to the Events tab and click Add New Function.

  - Select ActivateScreen, then choose the target screen.

  - Click Create to confirm.

Method 2:

    - Simply drag and drop the target screen onto the root screen.

## 4.Editing "Automatic" Screen (Screen_1)

### Add a Start Button

  - Insert a button from the Elements menu.

  - Right-click the button and select Properties.

  - Navigate to Events, click Add New Function, and choose SetBit.

  - Select the desired PLC tag from the PLC Tags menu.

  - Add another function, ResetBit, and select the same tag.

Use the same procedure to make the stop button

### Add a Running Indicator (Lamp)

  - Insert a rectangle from the Basic Objects menu.

  - Right-click the rectangle and select Properties.

  - Set the default color to red (indicating "not running").

  - Navigate to Animation > Display > Appearance.

  - Click Add New, select the Start Light tag, and set the active color to green when the tag is ON.

### Display Target Position (I/O Field)

    - Insert an I/O field from the Elements menu.

    - Navigate to Animation > Process Value.

    - Add the tag that represents the target position.

### Add a Navigation Button to Return to the Root Screen

Insert a button and use either of the two methods mentioned above to navigate back to the root screen.